Smart Connectivity and Fusion Research Proposal

Task-Oriented Multi-Modal Semantic Information Fusion for Integrated Sensing and Communication in 6G Edge-IoT Networks

Abstract

The evolution toward 6G introduces unprecedented requirements for ubiquitous connectivity and sub-centimeter environmental perception, driving the emergence of Integrated Sensing and Communication (ISAC). Concurrently, Semantic Communication reduces bandwidth requirements by transmitting task-relevant "meaning" rather than raw bits. However, realizing a fully integrated 6G architecture remains an open problem due to the modality gap—existing ISAC systems rely primarily on single-modal radio frequency (RF) sensing, which lacks contextual awareness in highly dynamic, occluded environments. This proposal outlines a concise, feasible research plan to develop an Integrated Sensing, Computing, and Semantic Communication (ISCSC) framework for high-mobility 6G edge-IoT networks. By employing an edge-centric architecture utilizing Transformer-based feature alignment and a Mixture-of-Experts (MoE) gating mechanism, the framework dynamically fuses visual optics and RF Channel State Information (CSI). This facilitates adaptive semantic resource slicing, maximizing transmission rates with the long-term objective of incorporating CRB-aware sensing constraints.. Preliminary small-scale experiments provide partial evidence for the feasibility of channel-aware semantic resource slicing. However, current radar-camera fusion results on a tiny real-world foggy sample do not yet demonstrate consistent superiority over camera-only perception, motivating further validation on larger multimodal datasets.

1. Introduction and Motivation

The transition to 6G Space-Air-Ground Integrated Networks (SAGIN) demands an architectural leap. Two distinct technological paradigms have gained paramount importance: Integrated Sensing and Communication (ISAC), which unifies data transmission and radar-like sensing within a shared spectral footprint, and Semantic Communication, which drastically reduces bandwidth by conveying only goal-oriented contextual intent.

However, the intersection of ISAC and Semantic Communication presents a formidable open problem. The heterogeneity of stochastic communication signals fundamentally clashes with the deterministic waveforms required for high-precision radar. Furthermore, current semantic architectures deployed on constrained Internet of Things (IoT) devices are severely fragile in dynamic channel environments and often operate in isolation from the physical layer's native ISAC sensing capabilities. This research posits that integrating multi-modal sensory data—combining RF CSI physical geometry with visual semantics—is the critical missing vector. This proposal addresses the systemic gaps by formulating a lightweight, feasible ISCSC architecture that dynamically balances physical-layer sensing CRB and application-layer semantic transmission rates.

2. Literature Review and the Open Problem

Recent peer-reviewed literature reveals localized advancements but a persistent systemic gap in multi-modal ISAC fusion.

ISAC Limitations: Network-level ISAC deployments are constrained by non-Gaussian interference generated by simultaneous communication streams, degrading classical Extended Kalman Filter (EKF) tracking algorithms. Recent hybrid ISAC-LSTM architectures remain reactive and ignore semantic context.

Semantic Communication Fragility: Traditional semantic encoders (e.g., Variational Graph Auto-Encoders) fail to adapt when channel states rapidly deteriorate. Although recent frameworks address computational bottlenecks, they underutilize inherently available RF environmental data.

The Modality Gap:Single-modal RF sensing is insufficient for 6G.Integrating multi-modal modalities (RF point clouds,optics)poses vast mathematical challenges.

The Open Problem: There is currently no unified, lightweight ISCSC framework capable of performing task-oriented, multi-modal semantic fusion that dynamically balances physical-layer sensing Cramér-Rao Bounds and application-layer semantic transmission rates via adaptive resource slicing in high-mobility edge environments.

3. System Model and Problem Formulation

Consider an Internet of Vehicles (IoV) edge server with a multi-antenna array serving K mobile IoT devices.

ISAC Transmission:

The transmitted ISAC signal matrix X over L discrete time slots combines semantic communication symbols (Sc) and radar sensing signals (Sr):

X=WcSc+WrSr

where Wc and Wr are the respective spatial beamforming matrices.

Performance Metrics:

Semantic Transmission Rate (Rsem): Defined as a function of the physical SINRk and the dynamically determined semantic extraction ratio Ckϵ(0,1] and its fidelity function Φ(Ck) :

- Rsem,k=Blog2(1+SINRk)∙Φ(Ck)

- Sensing Accuracy Bound: The fundamental limit is bounded by minimizing the trace of the Cramér-Rao Bound (CRB) matrix,Tr(CCRB(Wc,Wr)) .

- Joint Optimization Problem: The objective is to maximize aggregate semantic rates while penalizing sensing degradation, subject to power (Pmax), computational (Fmax), and latency (Tmax) constraints:

- maxWc,Wr,Ck,fkk=1KUk(Rsem,k)−λ⋅Tr(CCRB(Wc,Wr))

4. Proposed Multi-Modal Expert-Driven ISCSC Architecture

- To solve this highly non-convex formulation, the proposed architecture consists of four tightly integrated pillars:

- (1) CSI-Assisted Region of Interest (RoI) Extraction: The edge server utilizes Time of Flight (ToF) and Angle of Arrival (AoA) physical spectra from radar echoes to map a coarse physical RoI. This is fed back to the IoT device, filtering out up to 94% of redundant visual background prior to neural network processing.

- (2) Transformer-Based Feature Alignment Module (FAM): At the edge, a multi-head cross-attention mechanism fuses the highly compressed incoming visual semantic representation (Fv) with high-fidelity 4D RF sensing point clouds (Fs). This mechanism injects precise spatial CSI cues into the sparse visual features to interpolate lost data.

- (3) Mixture-of-Experts (MoE) Gating Framework: The fused multi-modal vector is routed through an MoE gating network utilizing a Softmax function:

- αm=exp(wmTz)m′=1Mexp(wm′Tz)

This dynamically assigns inferential authority among highly specialized Sensing, Communication, and Joint experts based on real-time environmental occlusion and channel entropy.

- (4)Task-Oriented Resource Slicing: An Alternating Optimization (AO) algorithm iteratively untangles the variables: optimizing semantic extraction ratios (Ck) given fixed beamforming, and then utilizing Semi-Definite Relaxation to update beamforming matrices (Wc,Wr) to minimize Tr(CCRB) while satisfying required SINR thresholds.

5. Experimental Results and Feasibility Analysis

A concrete experimental simulation was built utilizing Python and PyTorch. A fully runnable codebase is attached as a supplementary file to this proposal. The experiments consist of two small-scale public-data evaluations: a DeepMIMO channel-aware semantic slicing proxy experiment and a RADIATE tiny foggy radar-camera perception experiment.

Semantic Communication Reliability: The channel-aware resource slicing component was evaluated using a SER proxy derived from DeepMIMO ray-tracing channel quality.

DeepMIMO channel-aware slicing experiment:

- Fixed policy SER proxy: 0.3904

- Adaptive policy SER proxy: 0.3268

- Relative reduction: 16.28%

Multi-Modal Sensing Accuracy: Multi-modal perception was evaluated on the RADIATE tiny foggy sample using frame-level object-count regression error.
RADIATE tiny foggy radar-camera experiment:

- Camera-only MAE: 0.176

- Radar-only MAE: 0.566

- Fusion MAE: 0.402

- Best modality: camera-only

Analysis: As demonstrated, The current results partially support the channel-aware resource slicing component, but do not yet validate the full multi-modal fusion superiority claim. In the RADIATE tiny foggy sample, camera-only features achieved lower object-count regression error than radar-only and naive fusion features. Furthermore, the CSI-assisted RoI extraction remains a design objective and has not yet been validated in the current tiny-data experiments.The SER value is a proxy derived from ray-tracing channel quality rather than an over-the-air semantic communication measurement.

6. Conclusion

This proposal presents a feasible architecture for integrating semantic communication, RF sensing, and edge intelligence. Current small-scale public-data experiments suggest that channel-aware semantic slicing is promising, while the multi-modal fusion component requires further validation on larger synchronized datasets such as RADIATE full sequences, CARRADA, Multimodal-Wireless, or Sensiverse.

References

- Y.He, "Task-Oriented Integrated Sensing and Semantic Communications for Multi-Device Video Analytics," IEEE Transactions on Mobile Computing, pp. 1-14, Dec. 2025.

- Z.Yang, et al., "Integrated Sensing, Computing, and Semantic Communication with Fluid Antenna for Metaverse," arXiv preprint arXiv:2504.07656, Apr. 2025.

Karahan, S.N. Hybrid ISAC-LSTM Architecture for Enhanced Target Tracking in Integrated Sensing and Communication Systems: A Symmetric Dual-Function Framework. Symmetry 2025, 17, 1725.

"Sensiverse: A sensing channel dataset for multi-scenario and multi-mode integrated sensing and communication," 2025.

Y. Ren, et al., "Semantic-Empowered Integrated Sensing and Communication," IEEE, 2025.

"Target tracking in integrated sensing and communication (ISAC) systems via hybrid ISAC-LSTM architecture," MDPI, 2025.